H.C.B-A1432

DAF TRANSMISSION NEUTRAL POSITION ADJUSTMENT TOOL

2

1

Step:

Before using this tool, seal the opening of the transmission in case object falls into it.

- Put the central selector shaft in the neutral position, and then fix H.C.B-A1432-1 to the upper lid of transmission to align the control rod.

- Screw H.C.B-A1432-2 to the transmission. To start align the central selector shaft first.

- Use the same way to align the rest selector shafts.

- Used to adjust the neutral position of transmission.

- H.C.B-A1432-1 is for adjusting the control-rods in the upper part of transmission.

H.C.B-A1432-2 is for adjusting the selector shafts of transmission.

- Application: DAF Tiptronic truck (EURO 3.4.5).