H.C.B-A1606

SCANIA 340/380 (EURO 4) CRANKSHAFT FRONT/REAR OIL SEAL INSTALLER (PR SERIES)

3

4

2

1

1-2-1

1-2-25

| No. | Name | Description | Q’ty | Remark |
| --- | --- | --- | --- | --- |
| 1 | Installing base | D:ψ175x156L | 1 | M16xP1.5 |
| 2 | Fixing oil seal base | D:ψ133.2x171L | 1 | M20xP1.5 |
| 3 | Installing nut | L49xW296xH101.7 | 1 | M20xP1.5 |
| 4 | Front oil seal installing sleeve | D:ψ159.4x7.8L | 1 | d:ψ140.7 |

■Description: When installing the oil seal, you won’t damage the oil seal. With this tool kit will make

repair be more convenient and easier.

■Application: SCANIA 340/380 (PR Series)

■Oil seal genuine no.: 143 31 83

Front oil seal Installation:

Put the oil seal first, and then put no.4.

Screw crankshaft belt to appropriate place.

Rear oil seal Installation:

Hitch no.2 by oil seal. Screw 1-2-1 and 1-1-2 onto crankshaft.

Screw no.1, and then screw no.3.

Installing finished.

3

Please hitch new oil seal on no. 2

2

Screw the nut until finish installing.

Finish installing.